# Architecture and Implementation

“For every problem there is one solution, which is simple, neat, and wrong”

– H. L. Mencken

In order to examine the effectiveness of a stream-caching algorithm, it is first necessary to construct a continuous media server using that algorithm.  Although detailed simulations of stream caching algorithms are conceivable, they are never a substitute for observing the behaviour of a real-world system.  In this chapter, the prototype server architecture that was built for experimenting with caching algorithms is presented.

The chapter begins with a presentation of the physical architectural components.  The performance of these components is examined and the theoretical performance of the server architecture derived.  The following sections then cover the individual parts of the server piece by piece, namely: the file system; the continuous media file format; the network interface, its driver and associated network protocol structures (from link layer to physical layer); and ultimately, the cache and streams management controllers, whose operations tie the previously covered items together to form the complete network server.

Each section discusses, where appropriate, the underlying hardware construction and the issues involved in arriving at a solution.  Where several design options are described in a section, the section states the option that was taken.  The intention in all sections is to give the philosophy in the design and state the reasons why a particular implementation was taken.  Each section concludes with an analysis of that component to show that its performance is sufficient given the requirements of the server.

## A Base Architecture – Foundations

To successfully transmit video data to a client with sufficient temporal accuracy requires a processor or operating system with known highly predictable timing characteristics.  In particular, it must respond to real-time events, most notably the transmission of a frame of video, within a sub-millisecond time frame.  One such processor is the INMOS Transputer; while traditional Unix implementations give a good base on which to build, their real-time response is far too coarse for the successful implementation of a multi-user video file server.

The Transputer has been, up until very recently, unique in its employment of four high-speed serial links to allow the networking of multiple processors.  In terms of the video server, this allows a breaking up of process responsibility, a dedication of function and therefore the avoidance of overloading a single processor with all the many tasks required of the server.

The prototype architecture is arranged as illustrated by figure 4.1.  At the right of the diamond topology is a dedicated filesystem TRAnsputer Module (TRAM), and at the left, an ATM network interface TRAM.  The four TRAMs across the middle of the topology are responsible for the main data cache.  The Root TRAM is used for booting the server and for logging of system data and events as required by the server.  It also provides a console interface to the server.

Figure 4.1: The Prototype Server Hardware Architecture

### Theoretical Requirements

The primary objectives of the server are firstly to stream data from the disk subsystem into the cache, and secondly, to stream data from the cache to the ATM TRAM for transmission.  These two data paths are illustrated in figure 4.2 and are labelled as paths X and Y, respectively.  These paths are the result of the server being multiprocessor; the data transmission must be staged between the processors, and furthermore, the data must be prepared on the ATM TRAM for transmission to the network.  Figure 4.2 shows the ATM TRAM as receiving a block for transmission into a “Service Data Unit” (SDU) buffer, which is then segmented into ATM cells for subsequent transmission.

The data cache affords a higher outgoing transmission rate to the network than the disks alone would allow.  Hence, in figure 4.2, the SCSI TRAM does not receive all data requests from the clients, since some of these will be satisfied by cache data.  For the purposes of the following feasibility study, a cache-hit ratio of 80% was assumed.

Given the physical characteristics of the Transputer, the theoretical requirements of the architecture outlined above can be examined.   The following calculation was a ‘back of the envelope’ estimate performed prior to construction of the server; it is not intended as an accurate mathematical model of the prototype server.

Let:

| dm | = Maximum disk transfer rate (bytes/s) |
| --- | --- |
| ed | = disk event rate (Requests/s) |
| H | = cache hit rate |
| M | = cache miss rate = 1 - H |
| n | = number of video streams |
| sv | = average size of video frame |
| fr | = video frame rate (frames/s) |
| ef | = cumulative frame request rate(frames/s) |
| rhw | = hardware link transfer rate (bytes/s) |
| thw | = time of transfer of frame across a hardware link |
| nhw | = number of hardware links used for transfer |
| ta | = time of memory allocation for frame |

Figure 4.2: Data flows within the server – Disk to Cache and Cache to Network

Firstly, are the disks fast enough to support the required n users given the assumed hit ratio, and further, how fast do the disks need to be?

The cumulative frame rate is the product of all video streams:

ef=n×fr

The cache will catch a percentage of these requests, so the event rate seen by the disk is lower:

ed=ef×M

ed=n.fr.(1−H)			(4.1)

The actual disk transfer rate required, dr, may thus be calculated:

dr=ed×sv

dr=n.fr.(1−H).sv			(4.2)

where

dr≤dm

For each block transferred into memory from the disk, the block will be transmitted out on one of the hardware links at the rate of rhw bytes per second (Path X in figure 4.2).  However, in order for the block to be placed in main cache memory, it must first have been allocated a buffer.  The maximum rate of the disk and hardware link are fixed, giving an amount of time remaining per second from which buffers can be allocated.  This allocation time will also include any necessary freeing of other buffers where there is not enough memory on the initial buffer request.  In the following, the effect of the double buffer in the SCSI TRAM is ignored, since the worst case is the transfer of single blocks of data scattered over the disks.  In this case, the double buffering is defeated because moving the disk head breaks the transfer pipeline.  Furthermore, consideration of the worst case gives a safe basis for prediction of system performance.

ta=1ed−svdm+svrhw.nhw

ta=1n.fr.(1−H)−svdm+svrhw.nhw		from equation 4.1

Each Transputer link is rated at 20Mbps.  For each byte of data transmitted over this link, there is 1 start bit and 2 stop bits, that is, an overhead of 3 bits for each 8 bits transmitted.  Consequently, there is an effective throughput on each link of 20×8÷(1+8+2) = 14.55Mbps or 1.73MBytes/s.

Let:

sv=20 KBytesfr=25 frames/sdm=2 Mbytes/s =2097152 Bytes/srhw=1.73 Mbytes/s =1814000 Bytes/sn=10H=0.80

It is assumed that an 80% hit ratio H on the cache can be achieved.  Thus:

ta=7.4ms

This proves that the proposed architecture is feasible; it should be possible to build a memory allocator at least an order of magnitude faster than this.

The above is, however, pessimistic: for any of the four cache TRAMs, we may assume that a frame request can be made, the buffer allocation made for it, and the data then placed into the main cache buffer.  It will take far more time for the SCSI TRAM to fetch the data and transfer it to a buffer, than to make the allocation of that buffer.  Hence, for the cache TRAMs, only thw is effectively incurred where

thw=svrhw

For a cache hit, data is output over the hardware link to the ATM TRAM; for a miss, the data must firstly be fetched from the SCSI TRAM.  Thus, a cache miss is at least twice the cost of a cache hit.  For a miss, the cache will be kept waiting while the frame is fetched from disk.  However, since the server is dealing with video streams, the frames can be obtained in advance of transmission through the ATM TRAM; to the ATM TRAM, the cache success (i.e. delivery of data) rate could potentially be 100 percent.

Given the time taken by the cache in transmitting data to the ATM TRAM per second, sufficient idle time must exist to allow the server to perform allocations, disk fetches and other cache maintenance operations.  Thus, does the server have sufficient time outside of cache to ATM transmissions for other tasks?

Let:

nt= the number of transmissions the cache deals with per secondnc= the number of TRAMs making up the cache

then

nt=efnc

If nc = 4, and ef = 250 (10 users at 25 fps):

⇒ nt=2504nt=63 link transfers per second per TRAM

Taking these transfers as occurring per second, we define

ca= Cache activityca=nt.svrhws or %(since per second)

In the above case,

ca=63×201.73×1024ca=71.1% busy

In the above, we have  290 ms remaining per second.  This time is available for waiting on the SCSI TRAM or for any allocations that may be required.  Here too, the architecture looks feasible.

For the ATM TRAM, the rate at which data can be transferred over the hardware links is limited (Path Y in figure 4.2).  Given this maximum link rate and the video bit-rate, how many users can be supported by the server?  If each hardware link can run at 1.73 Mb/s, then for all four links:

Rhw= total data rate of all available links=rhw.nhw =1.73 MBytes×4Rhw=6.92 Mbytes/s =58 Mbps

The maximum number of streams that can be handled can be calculated thus:

n=Rhw.Ehwrv

where

Ehw= Efficiency of hardware links achievedrv= Rate of video per second =sv.fr

It is unlikely that the links will be kept fully busy; it is assumed that all four links can be kept partially busy and that they are serviced simultaneously – this may not be possible in practice.  Note that the following calculation assumes a video bandwidth of 2Mbps  250 KBytes/s (MPEG streams used in practice are 1.5 Mbps; we consider a worse case here).  If the four link engines are only 80 percent efficient:

n=7256 KBytes×0.80250×1024

n=23  streams

Thus, the cache to ATM TRAM transfer paths are capable of the requirements that will be made of them.  From equation 4.2, it is possible to check that the disks are fast enough for the calculated maximum number of users:

dr=n.1−H.rv

dr=23.1−0.8.250×1024

dr=1150 KBytes/s=1.1 Mbytes/s

Theoretically therefore, the architecture should be able to support the desired twenty simultaneous users. Furthermore, the above calculation suggests that the disk speed required by the server is within the performance envelope of any standard disk, provided that the cache hit rate can be maintained.  With respect to the required cache hit rate, equation 4.2 can be arranged thus:

H=n.rv−drn.rv

H=20.250×1024−2×1024×102420.250×1024

H=0.59=59 %

The above calculation takes the maximum disk rate as 2Mbytes/s.  Hence, the architecture requires a cache hit rate of 59% in order to succeed as a video server.

### Practical Realities – Building an Infrastructure

In order to test the achievable rate of data transfer from the network and file system TRAMs to the data cache and vice versa, an initial infrastructure was created.  The test-bed transferred blocks of ‘fake video’ data from the file system to the cache to the network adapter, and separately, back in the other direction.  Unless the theoretical requirements could be satisfied, there would have been little point in continuing with the construction of the video server using the given architecture.

It was clear that in order to succeed with the Transputer based architecture, the links would need to be driven at near 100 per cent.  Furthermore, if the time between packets of data outbound on a link were to be minimised, the (n+1)th packet would need to be prepared and ready for transmission before the nth packet completed transmission; that is, double buffering would be necessary on any outbound data packets.

With the Transputer link limited to a user bandwidth of approximately 15Mbps (1.73 MBytes/s), it was clear that all four links would need to be engaged simultaneously when fetching data from the file system to the central data cache.  This realisation was crucial in the design of the layout of the contents of the data cache; each successive data block would need to be transferred to a different cache TRAM than the previous three blocks if all four links were to be utilised.  Consequently, the cache addressing is arranged such that the least significant two bits of a file's block offset (relative to that file's own start block), are the selector for which cache TRAM should contain that data.

### Basic Performance Requirements in the Server

As the data transfer time was critical to the success of the base architecture, memory copying of file data between processes on a single TRAM needed to be avoided.  Thus, blocks of data are passed between processes by reference to their block offset within the block array from which they were allocated; this is effectively the block's address, although Occam explicitly forbids pointers – hence the alternative approach.

To add to this complexity, each physical Transputer link allows the placement of only one Occam channel in each direction of flow.  Thus, a multiplexor/demultiplexor process pair is needed at each end of each bi-directional link.  In addition, each outgoing compound data structure has an associated scheduling overhead for each protocol data unit within it.  For example, the protocol ‘BOOL ; INT ; [5]INT’ contains three protocol data units, namely, one boolean, one integer, and a fixed size integer array of five; this requires three CPU scheduler cycles to transmit or receive across a link.  A separate transfer for each protocol component is required because each part may have independent (that is, non-contiguous) memory addresses from the other parts; a ‘memcpy()’ operation is required for each part.

The larger each transfer packet, the fewer packets per unit time; similarly, the fewer the number of protocol units per packet, the lower the CPU overheads per packet.  Collectively, the lower the scheduling overhead, the greater the resulting bandwidth and CPU time available.  Consequently, multiple protocol data units are packed into a single outgoing buffer, including the multiplexor's identifying tag used by the demultiplexor to determine which process should be forwarded any received data packet.  Of course, the media data is the exception here – the multiplexor sends the tag separately from the data such that a large memory copy from the cache into the multiplexor is avoided.

### Analysis of the Base Architecture

At this early stage in the server's development, only the infrastructure had been written.  Consequently, the only data paths that it was possible to exercise were between the file system and the cache, the network TRAM and the cache, and vice versa in both these cases.  The purpose of this evaluation step was to check that the data paths were fast enough to satisfy the theoretical requirements.

A double buffered transfer over any single unidirectional link was found to produce either 11Mbps (1.4MBytes/s) or 14Mbps (1.7MBytes/s).  The pattern of slower links was found to correspond exactly with those hardware links that had been connected up via a C004 link routing processor; the others were all direct link connections.  Having noted this pattern, it was decided that for a ‘release’ version of the server, all the links would need to be directly connected such that maximum performance were achieved.  For the purposes of experimentation however, it was felt that this was not necessary.

When engaging all four hardware links in a sustained transfer of data blocks via the link multiplexor/demultiplexor process pair, a sustained transfer rate of over 40Mbps (4.8MBytes/s) was observed. These transfers used a block size of 4096 Bytes such that CPU scheduling overheads were minimised, this number being chosen on its convenience in respect of both file system logical block size and ATM network packet size.

Given the theoretical requirement that all the links be near 100% utilised, and that the four links together could only be driven as fast as the slowest link, the infrastructure was seen to be more than satisfactory and development continued.

## A File System For Continuous Media

A file system for the storage and retrieval of multimedia data, in particular video files, was needed that gave optimal bandwidth as needed for multimedia applications, and that gave long-term reliability and crash resistance.

The recovery operation of the file system needed to be quick as the initial project prototype that would be using it would likely crash frequently (it did).  It was important that the file system be recovered quickly after a crash if the project was not to be held up by frequent repairs.

The underlying operational details of any file system are hidden from the user to whom each file appears as a stream of bytes and the physical disk devices appear as a single storage facility.  The file system therefore presents an interface to the user allowing byte level manipulation of an arbitrary collection of named files.

Each file has attributes describing its symbolic filename, owner and permission information for security, plus timestamps and size information as per the Unix BSD4.3 file system [MJL84].

In order to work out how many files were likely to be stored, it was assumed that the smallest clips were likely to be in the order of 5 seconds.  It had been found previously that frame size averaged approximately 20KBytes per frame for MJPEG footage.  Hence, assuming an average frame rate of 23 (25 is desired, but we consider a worse case) then:

ts=5s×23fps×20Kbytests=2300Kbytes

Thus, over one of the 900 Megabyte devices used in the prototype server, there was a requirement of 400 large files, in contrast to the Unix assumption of thousands of mainly very small files.

### Block Allocation – The Principle of Locality

The raw disk had been found capable of transfer rates in the region of 2.2 MBytes/s, but with the introduction of a file system, the effective transfer rate would inevitably be degraded.  The file system needed to layout its data in such a way that the resulting transfer rate was close to the raw rate of the device in question; the speed of data access was a priority in the design.  The approach taken was that of data locality, that is, storing related data within the same area of the disk.  In this way, the seek time incurred in reading these data is minimised.

For all files, each has a header that describes the attributes of the file and the location of its data on the disk.  Thus, access to the file necessitates access to the header.  To maximise transfer rate, the seek-time between the acquisition of the header and the file data must be minimised; the file header must be located on the disk local to the file that it describes.  The file header is called an f-node [Dun89], similar to the Unix i-node.  The contents of the nodes however, are not the same.

The maximum transfer rate is achieved by accessing the disk sequentially.  The files should therefore be laid out sequentially on the disk.  A contiguous file system however is very inflexible, so each file is made up of several extents.  The f-node therefore records the starting block number and length of each extent, the extents being sorted in the order of increasing logical file address.  To access a specific location in a file requires a search of the extent descriptors in the f-node.  This is a fast operation provided that the file is not made up of very large numbers of extents, ie. is highly fragmented.  It is the responsibility of the block allocation strategy to prevent such excessive fragmentation, so making the cost of traversal insignificant.  The benefit of the approach is that a large file may be described by a few extents, as opposed to a Unix like i-node which would have one entry for each block that the file occupied; for a large file this becomes significantly larger than the f-node equivalent, even when highly fragmented.  Twinning this strategy with a good allocation mechanism and a defragmentation utility should give better performance than the i-node alternative.

Seek times can be lower if more file header areas, or bands (See figure 4.3), are available since the distance to the nearest f-node band will be lower on average.  The disadvantage of many node bands however, is more wasted disk space due to unused f-nodes and smaller contiguous areas forcing large files to be made of up more extents.  Node banding therefore gives much less benefit than in the Unix environment for which it was designed and instead becomes a source of fragmentation in data files; consequently, the approach was ruled out.

Figure 4.3: Demonstration of node banding to achieve data locality

When considering the disk as a whole, many files need to be described, all of which use the principle of locality.  Not all files can be located near the single central f-node band, so the files are spread over the disk into several regions.  The cost of fetching a file header therefore increases as the location of a file moves away from the single f-node band.  Due to the size of the files concerned and the duration for which each is held open, the increase in access time should be insignificant relative to the amount of data accessed before the file is closed.  In fact, this cost is obviated by permanently caching the entire f-node band.

### Variable Track Disk Format

In large disks, the number of sectors per track decreases from the outer to the inner cylinders.  This has the effect that the raw transfer rate is higher on the outside cylinders and decreases toward the inside cylinders.  In order to reduce the average seek-distance (and hence seek time), the allocation of files in Unix BSD4.3 occurs from the central cylinder outwards, i.e. from half radius, in both directions.  Thus, the average seek distance is only ¼ (½ of half) of the disk radius.  Disk optimisation can place the most used files around the central cylinder such that disk layout mirrors the distribution of access – the disk seeks are therefore minimised.

Unfortunately, the central cylinder does not have the greatest transfer rate, so the faster outside tracks tend to be largely unused.  Thus, the server's file system locates its f-node band at and allocated around the first third of the disk (from the outer cylinder) in order to take advantage of the greater transfer rate.  Disk optimisation utilities could likewise use this strategy so that the most used files are located around the f-node band on the fastest area of the disk.

### Reliability and Crash Resilience

In addition to ordering the writes to the disk to ensure that directory and f-node data cannot become inconsistent, the node band is used for all file and directory f-nodes.  This allows a fast recovery from failures since the location of the file and directory headers is known, and thus a lengthy scan across the entire disk is avoided.  A valid f-node block is identified via a unique signature, so the probability of attempting to recover invalid or corrupted files is minimised.

The location and size of the node band are stored in the superblock of the device, so that on recovery, only the blocks indicated need be examined.  Should all superblock copies be destroyed however, then the entire disk would need to be scanned for the signatures and other signs of structure data.  This expensive procedure is eliminated by locating the band after a known interval from the first sector.  The interval is calculated as the total number of blocks on the device divided by three (rounded down).

To prevent loss of data due to overwriting in an inconsistent filestore, a flag in the superblock indicates whether the file system was cleanly shutdown or not.  If the flag shows the filestore to be ‘dirty’ then the recovery procedures, a tailored version of ‘fsck’, are invoked automatically.  In practice, this proved to be a valuable debugging aid while perfecting the file system code.

### F-Node Contents

The contents of an f-node are shown in table 4.1 below.  The reference count refers to the number of directory entries that point to a file.  The mode field indicates if the f-node describes a directory or a file, its permissions, and further, whether or not the last index entry is a direct extent pointer, or an indirect pointer to another node.

Each extent is described by the initial sector and the length of the extent.  If the logical block size is assumed to be at least 4KBytes, with a 2-byte length field, the maximum extent length is at least 216 - 1 = 65535 blocks = 262140KBytes or 256MBytes.

A single 1-Gigabyte device has 218910 sectors of 4096 bytes each.  It is not enough to use 2-bytes as a sector number, and three byte fields are difficult to manipulate; the physical sector address range requires 4 bytes for a 1 Gigabyte device.  Thus, each extent requires 6 bytes.  In general, six extents should be enough to describe a file, giving an f-node size of 6 × 6 + 26  64 bytes (word aligned).  Increasing the size of a logical block to 16K would mean that the necessary addressing range decreased from 0..218909 to 0..54727 and further that only 2 bytes are required per block address, allowing 9 extent entries.  However, this approach limits the maximum device and volume set capacity supported to 1 Gigabyte.  In the general case however, 6 entries should be more than enough for the average sized file, especially as each file will ideally be stored as one, or at most two extents.

Table 4.1: Contents of F-node

| Field | Size/Bytes |
| --- | --- |
| Generation | 2 |
| Owner uid | 2 |
| Owner gid | 2 |
| Permissions | 2 |
| File size | 4 |
| Time created | 4 |
| Time last accessed | 4 |
| Time last modified | 4 |
| Mode | 1 |
| Reference count | 1 |
| Total | 26 |

If the number of direct extents are insufficient to describe a file, then the last entry becomes a pointer to another f-node.  Thus, the new f-node contains 6 further extents, with the mode byte indicating that the f-node is an indirect block and an f-node pointer to a further indirect block if required.  The f-node chaining can continue indefinitely such that highly fragmented files may be described, though this is clearly undesirable.  After an f-node becomes an indirect block, the majority of the fields in the f-node become redundant and are used instead as pointers to the parent f-node to assist the recovery operation in the event of a crash.

The addressing of f-nodes is not done by either physical sector numbering or logical block numbering, since several f-nodes exist within one sector.  The f-nodes are addressed as an array and require only 16-bits giving a maximum of 65535 f-nodes in total.  Disregarding directories and indirect blocks, this gives over 65000 files addressable by the file system per device.

Considering a 512 byte sector, each of the 8  62 byte f-nodes leaves 2 bytes unused.  This area is used to store a unique f-node signature such that during recovery, valid f-nodes can be identified immediately.  The signature occupies the first 2 bytes of each f-node and thus increases the size of an f-node to 64 bytes; this results in no unused space within an f-node block.

### Directories

The root directory is defined by the superblock.  The superblock points to an f-node which is itself marked as a directory f-node via the mode byte.  The operation of the extent entries is exactly as for data files, but the data contained are information on the file structure.  The directory entries are structured as a list of arbitrary length file names and f-node pointer pairs; the indicated f-node contains the header information for the named file.    Each entry also contains a ‘space used’ field so that it is known how much space is unused between two adjacent entries, and new entries can be allocated based on this information.  Thus, in an empty directory, the size used field is the logical block size.  No directory entries may span over a logical block boundary.

When the file system is first initialised, the first f-node entry is reserved for the root directory.  On recovery, the first f-node is therefore known to mark the root directory of the file system.  Similarly to f-node blocks, the first 4 bytes of a directory block contain a unique signature indicating the block to be a valid directory.

### Cache Management for Optimisation

In order to access data within a file, the f-node of the file concerned needs to be fetched.  This occurs for every data access, so there are two accesses to the disk for each data access.  In order to improve performance, the f-node access is eliminated simply by caching the f-nodes.  Thus, for all such files, there is only one access made by the disk device for each read request made.

A similar argument applies to directories; their f-nodes are cached with the data file f-nodes.  In addition, an entire directory (consisting of one or more data blocks) is read with a single disk access, and is likewise cached so that frequent operations on such a directory are optimal.  If all f-nodes are already in the cache, then directory access is limited only by the time it takes to read the directory block(s) from disk.  Once the blocks are cached however, subsequent directory operations will hit the local metadata-cache and require no disk access.

### Free Block Mapping

The allocation of disk blocks is controlled by two bitmaps located directly below the superblock in the lower track of the inner cylinder.  Since the bitmaps are rarely read/written, and are not critical to the integrity of the disk data, they are placed on the slowest area of the disk so that faster regions remain available to disk data where transfer rate is more valuable.

The two maps are for f-nodes and data zones (logical blocks).  For the zone map, each zone on the disk is represented by a single bit, such that every eight zones require one byte of data.  For the f-node map, each f-node is also given one bit, so that each 64 f-nodes require 8 bytes.  For a disk with 8 (4096 byte) f-node blocks this gives 8  8 = 64 bytes.  In fact, the last block of the zone map has a large number of bytes unused.  Thus, the f-node map does not have a block reserved for it alone; the f-node map follows the zone map and thus shares its last block.  This has the advantage of probably saving one disk block access when flushing the maps.  In terms of disk space, this is an insignificant gain, though in terms of meta-data caching an extra block of f-nodes or a directory block is significant.  The cost of flushing the maps is also reduced.

Since both maps are very small, they are made memory resident to achieve maximum performance.  Both maps are flushed to the disk during periods of relative inactivity, and at system shutdown.  In addition, they are also flushed after a time-out period during heavy disk access, such that the flush is not indefinitely postponed.  The current file-system flush timeout period is set at thirty seconds.

When the disk is first initialised, the free-maps will be largely zeros.  The zone map marks where the superblock(s), free-maps, and the f-node bands are located.  The f-node map marks the root directory.  Hereafter, the map is used as the basis for allocating f-nodes or zones during write operations.  Should the file system become corrupted, then the free-maps are rebuilt from the f-node and superblock data; they are not used again until the file system has been recovered since illegal f-node or block reusage is inevitable – this is particularly so if the free-map has not been flushed to disk before a crash and after a long series of write operations.

### The File System

It is the responsibility of the superblock to describe to the file system manager how the disk is divided up.  Generally, it must locate its root directory f-node, specify the zone size for the device, point to the free-map, and so on.  The superblock is thus critical to the successful operation of the file system and is replicated on a second location on the disk.  The primary copy is located on the first physical sector, which is on the upper surface and the outermost track.  This area is the fastest on the disk; the superblock is placed here so that marking and unmarking of the ‘dirty’ bit can occur as quickly as possible prior to any data writes onto the disk.  The secondary block is located on the last sector of the disk, which is on the lower surface and innermost track.  Since it is rarely referenced besides start-up and shutdown, this leaves more valuable disk areas for data.  Thus, if the primary superblock becomes irretrievable, the secondary should remain available.  In addition, if other physical devices are available, the superblock of each can be replicated ahead of the free-maps of every other device.  Thus, if both primary and secondary superblocks are destroyed, one of the other devices should contain a valid copy.  A diagram of the server's file system is shown in figure 4.4.

Figure 4.4: General Structure of the File System Volume

On start-up, the superblock of each device is read into memory where it remains until shutdown when each is flushed back onto its respective disk.  Both the primary and secondary blocks are updated on shutdown, plus the copies of any other devices' superblocks.  After the initial superblock read, any write that occurs will set a flag in the superblock to indicate that the superblock was read in, but has not been shut down.  As this operation occurs prior to any writes, if a session occurs with no writes, the superblock will still indicate that the filestore contents are consistent and time is saved after a crash as no recovery operation need be invoked.  When the system is shut down, or the cache contents are flushed, this flag is again altered to indicate that the file system is in a consistent state.  Thus, if the file system should crash after any series of writes, the file system manager will find the ‘dirty’ flag on boot-up, will know that the file system is possibly inconsistent, and may invoke appropriate measures.  The date and time of the last ‘file system check’ are recorded as part of the superblock.

### Volumes and Volume Sets

From the users point of view the file system appears as a single logical device.  To achieve this illusion, each device has a unique volume label and serial number.  Within the superblock of the root device is a list of volume labels and serial numbers identifying which devices make up a particular volume set, plus a structure name, which identifies the set as a whole.  This is a principle used by the VMS Version 5.2 file system [McK90].  The root device has the relative device ID of 0.  The remaining volumes in the set are identified by increasing ID in the order in which they are declared.  The resulting logical block numbers contained by the f-nodes therefore contain the device ID within the top byte of the 4 bytes.  Thus, each device is limited to 24-bit block descriptors – a range of 0..16777215, which is not a problem (this is 8192 Gigabytes on a device with 512 byte sectors and blocks).  The superblocks of mounted devices indicate that they are part of a volume set, and specify the volume label and serial ID of the root device.  The f-node numbering of a volume set is linear, the address space of the (n+1)th device occurring after the nth.

It is emphasised that a volume set is a tight coupling – once the devices are formed into a set, removal of any one of the devices will render the file system useless without a tool to make the remaining volumes in the set consistent.  By a similar token, corruption of one of the devices affects the whole set, and the in-built fsck utility will attempt to clean up the set as a unit.

The allocation of blocks for files can thus occur across any of the volumes in the set.  However, the allocation strategy tries to prevent files from spanning across multiple disks, the exception occurring when a device cannot hold a particularly large file.  This should improve reliability since if one disk is destroyed, not all files are affected; only those on the device concerned are lost.  A further barrier to disk striping is the way the files are stored in f-nodes, namely, by extents.  Striping would result in each file f-node containing many extents, each extent on a different device to the next.  This problem could be overcome by having a logical sector numbering scheme where each x sector group is placed on the next device to the previous group and so on.  Thus, a single large extent can exist over all volumes within the set.  This would however, further constrain the set in that the removal or destruction of any one device will ruin all files on the set.

In using a fixed distance to the f-node band, the criticality of the superblock is reduced since it does not need to record the band's location.  The root directory can be located by indexing the first f-node of the f-node band, the device's format can be obtained by sending SCSI commands, and backups of other device superblocks identify themselves.  The file system is therefore not dependent on the availability of the superblock and is more reliable as a result.  All of the above information is however, recorded in the superblock for completeness.  The only information that cannot be recovered directly is the number of blocks per data zone.  Without this, the f-node indexing information is useless because it indexes zones, not physical sectors.  In fact, all of the above locations need not be fixed; however, until the file system becomes stable beyond doubt, the default locations are being used to assist recovery.

### Analysis of File System Performance

A discussion of the testing of the file system is deferred until the section on the implementation of NFS.  The sections between this and that of NFS cover the necessary components to complete the prototype as an NFS server.

## Implementing AAL5 on the UKC ATM Network TRAM

Using the in-house ATM network TRAM designed by Tripp[Tri95] and the cell driver designed and build by Prof P. Linington, it was possible to build upon the cell I/O routines in order to create higher level protocols, and hence enable basic communication with other network entities.  In particular, the ATM Adaptation Layer (AAL), equivalent to layer two of the OSI protocol stack, was a prerequisite to the development of protocols such as IP and UDP.  The next section outlines the ATM hardware structure and requirements, briefly describes the structure of AAL and goes on to discuss its efficient implementation on the Transputer.

### Hardware and Hardware Driver Issues

The ATM hardware board developed by Tripp[Tri95] comprises a T805 Transputer coupled with three Xilinx FPGAs, which drive the underlying network adapter hardware.  Each of the FPGA designs provides a set of memory mapped hardware registers that allow the T805 to operate the adapter hardware.  In addition, the Transputer memory space, which is located at address 0x8000000, is shadowed from the address 0xC000000.  The shadow memory space allows the T805 to exchange data in the main program memory-space with data in the ATM hardware FIFOs.

Each ATM cell consists of 53 bytes, of which 48 are the transport data and 5 bytes are network cell header.  The network header contains the VPI/VCI address pair, a cell loss priority bit, a payload type field, flow control information and a checksum.  In addition to the network header, there is also a set-up header, which contains behavioural directions and flags for the ATM hardware and a cell timestamp.

#### Cell Transmission Modes

In the transmission of cells onto the network, the Xilinx firmware supports three cell timing modes: immediate, absolute and relative cell timing.  The ATM hardware maintains a 16-bit clock using a tick rate of 80ns, giving a clock wrap time of 5.2 milliseconds.  The firmware treats the ½ clock period before the current time as in the past and the ½ period after the current time as in the future.  Thus, a cell in absolute mode with a transmission timestamp in the past is transmitted immediately.  Furthermore, a series of absolute timestamps that are in the past will cause back-to-back cell transmission at maximum cell rate.

In relative mode, a cell is timed relative to the previous cell submitted and the timestamp in this case is specified as zero-relative from the transmission time of the previous cell.  In firmware terms, the absolute timestamp of the previous transmission is recorded by the firmware and the relative transmit-timestamp calculated from this.  Hence, relative timing builds on top of the absolute timing mode and each relatively timed cell uses absolute timing mode with a calculated absolute timestamp internally within the firmware.

Care must be taken in both cell-timing modes that the absolute timestamp is presented to the hardware within ½ a clock period of the previously submitted cell, if that cell was transmitted.  If the timestamp, once submitted to the firmware, falls within this ½ clock period, there are two possibilities.  Firstly, if the timestamp indicates a time in the near future, the firmware waits for the necessary period.  Here, the cell is correctly timed relative to the previous cell (early submission).  Secondly, if submission to the firmware is late relative to the timestamp, it will appear to be due for transmission in the past, so causing an immediate cell transmission.  Alternatively, if the CPU timing is such that the absolute transmission timestamp falls further outside of the ‘past’ ½ clock period, then it instead appears to be due for transmission (relatively) far into the future.  In such a case, the ATM adapter stops transmitting for up to 2½ milliseconds.

At the time of writing, immediate cell transmit mode is not implemented.  Its purpose was as the first cell in a block of relatively timed cells, such that a new transmission after more than half a clock period does not risk stalling the transmitter.  However, the absolute cell-timing mode must instead be used by using the ATM clock’s value for the first cell to be transmitted.

#### Cell Structures

The two byte transmit timestamp in the set-up header are coupled with a further two bytes, which are used to extend the received timestamp on cells.  A dummy-cell indicator flag in the set-up header indicates that the ATM hardware clock has wrapped.  Thus, the software driver is able to detect the wrap and track the upper word of the ATM clock, and furthermore, to generate the full 32-bit received timestamp of all cells.

Together with the 4 byte timestamp, cell-timing mode byte, ‘wrap’ byte and network header, the ATM cell length in main memory is 53+6 = 59 bytes.  However, long-word alignment is maintained in both header and data sections in order to facilitate set-up and access to the cell headers by the Transputer, thus rounding the cell length up to 64 bytes.  Each cell therefore has a cell-header of 16 bytes, followed by 48 bytes of received, or to-transmit data.  Consequently, the received or to-transmit data are segmented into sections of 48 bytes, each separated by 16 bytes.  This is illustrated as follows:

| 16 | 48 bytes | 16 | 48 bytes | 16 | 48 bytes |  |
| --- | --- | --- | --- | --- | --- | --- |
| hdr | data | hdr | data | hdr | data |  |

The transfer of data into or out of the hardware FIFOs is achieved by reading the shadow address corresponding to the header of the first cell to be transferred.  The number of cells to be transferred is specified in the length register of the Xilinx that owns the FIFO.  The hardware uses an access page size of 1024 bytes, so at most 16 cells may be transferred per access.  The software driver employs an ISR that services the Transputer event pin; the ATM hardware uses this pin to indicate state changes within the adapter.  In addition to the event pin, the ISR uses the hardware registers to determine the reason for the interrupt and the actions that it needs to perform.

The software driver written by Linington is briefly described in the following sections.  The cell driver is responsible only for the driving of the raw ATM hardware from the Transputer’s main memory, and to build the 32-bit precision timestamp in received cells.  However, it is not responsible for the set-up header, network header (5 byte cell header), or the contents of the data segment (48 bytes) in each cell.

### Implementation Issues

The initial input and output mechanisms developed by Linington allowed ATM cell level transfers to occur; the construction and interpretation of the cells transferred are entirely the user's responsibility, most notably the cell header contents.  Figure 4.5 shows the basic design of the I/O system using Occam Model notation.

Figure 4.5: ATM Cell I/O Process Diagram

The cell.io driver process is responsible for the transmission and receipt of ATM cells to or from the network.  For efficiency reasons, cell transfers are batched together so that the ATM hardware sends or receives multiple cells while the Transputer continues with other processing.  Memory space for cell I/O is obtained from the two cell.buffer routines which maintain a cyclic cell array with queuing behaviour.  The purpose of each call to the cell.buffer is twofold: i) to return processed cells, which will have been read from or written into (if any); and ii) to request a number of cells for reading from or writing into as appropriate.  As said above, the reader from or writer to the cell buffers is allowed to request more than one cell buffer for use such that generation or processing of cell traffic may occur faster due to the decreased cell.buffer process interaction and consequent reduction in CPU scheduling overheads required.

As with the cache TRAMs, the multiplexor/demultiplexor processes pass I/O blocks by reference so that memory copying is avoided.  However, when placing data into or out of the cell buffers, segmentation or reassembly is required.  Consequently, at this point, a memory copy is unavoidable.  To this end, the Transputer instruction MOVE2D was employed to both segment and reassemble data.  The MOVE2D instruction is intended for windowed graphics applications to copy bitmap data from non-contiguous screen memory (the lines of data in a window bitmap) into another contiguous (or non-contiguous) area.  For example, to copy a picture from a back-buffer into a visible screen window, or move the window on the screen.  Thus, in the segmentation of an SDU buffer, the SDU is equivalent to a bitmap back-buffer, and the cells lines of 64 picture elements across; the SDU data is copied into the rightmost 48 bytes of each line – equivalent to a window.

### AAL Type 5 Structure Overview

The requirement was for a layer on top of the cell.io mechanism which allowed for variable sized data transfers to a given destination or from an arbitrary source, without the need to know the layout of the ATM cell header or be concerned with the Segmentation And Seassembly (SAR) of user data to and from ATM cells.  The ATM Adaptation Layer (AAL) provides precisely this service, though the server is concerned only with Type 5 AAL.  The structure of AAL Type 5 is as shown in figure 4.6.

As seen in figure 4.6, the AAL structure has AAL and ATM (cell level) Service Access Points (SAPs), and is subdivided into two main parts: the Convergence Sublayer (CS); and the Segmentation and Reassembly Sublayer (SAR).  The Convergence Sublayer is further subdivided into the Service Specific Convergence Sublayer and the Common Part Convergence Sublayer.  In the server's case, the SSCS is null and bridges the CPCS primitives directly onto the AAL primitives.

The CPCS sender accepts a payload from AAL-UNITDATA and appends a trailer which includes a CRC for the AAL-SDU.  The SAR sender then has the responsibility of segmenting the CPCS-PDU into ATM cells and sending them onto the network with the ATM-UNITDATA primitive.

The SAR receiver receives an ATM-UNITDATA indication when a segment of data has arrived.  The SAR then signals the CPCS receiver with the cell data so that it can be appended onto the appropriate receive buffer or a new buffer allocated.  Once the last segment of a block of user data has been received, the CPCS performs a CRC check and, if successful, delivers the data to the upper layer with an AAL-UNITDATA indication.  The reassembly process is thus co-operative between the SAR and CPCS.  For more detailed information on AAL, the reader is referred to [ITU93].

Figure 4.6: Structure Diagram of ATM Adaptation Layer 5[ITU93]

### Design Objectives

The original objectives were to be able to build further protocols on top of the AAL5 SAP; in particular, IP, UDP and NFS protocols needed to be implemented.  The ATM cell I/O system had been shown to be capable of its full 100 MBits/s transfer rate, which was far in excess of the bandwidth required for the intended application, and faster than the theoretical maximum rate of the 4 Transputer hardware links in any case.  However, with the performance of the network connection being critical to the application, the implementation needed to allow for IP control flows of around 10 Mbits/s, and AAL5 video data flows across the Transputer hardware links at near full link bandwidth (approximately 50Mbits/s).

Due to the size and volume of the information being transferred over the network, it was vital that Transputer memory copies be minimised in the transfer of user data between the layers of AAL.  At a bare minimum however, memory copies had to occur between the SAR and CPCS layers due to the need to segment the CPCS-PDU into 48 bytes per 64 byte ATM cell and vice versa on reassembly.  This process remains a notable critical section in the implementation of AAL.

Since the AAL5 video data transfers needed to be optimal, the IP and NFS paths not having such strict real-time constraints, only partial adherence to the AAL recommendation was considered.  Namely, avoidance of generation and processing of AAL block CRCs are considered.  It was for this reason that the CPCS trailers were included in the Continuous Media File Format; to generate the CRC for each trailer in real-time for every block of every frame is simply not feasible – it would be far too slow.  In fact, this results in conformance to the AAL5 specification.

The complete functionality of AAL did not necessarily need to be implemented solely on the ATM TRAM; it was possible to use the four cache TRAMs for part of the data processing.  Using the four cache TRAMs had the advantage that the ATM TRAM could dedicate its CPU time to the transmission and receipt of ATM cells and little besides, so that the efficiency of the ATM TRAM was optimal.  In practice, this turned out to be essential, with much effort expended in optimising the AAL datapaths.  The reader will note in the following sections that a single TRAM AAL5 implementation is possible with the AAL design presented.

### Implementation Considerations

The sender and receiver sides of AAL are differentiated as each has different responsibilities from its counterpart.  A simplified process diagram of AAL is given in figure 4.7; the actual implementation is given in more detail in the following sections.

The SDU.Buffer process manages a pool of Service Data Unit (SDU) buffers that other processes may request or return.  On the receive side, the CPCS requests SDU buffers for reassembly of SAR-SDU's into a CPCS SDU, which once assembled, is passed up to the AAL receiver.  The write controller returns the SDU buffer once the data are transferred across one of the Transputer hardware links.  To transmit a data block, the read controller requests SDU buffers, fills them with data from over a link, and sends them on to the AAL sender and so on down the layers until SAR sender frees the SDU buffer once segmentation is complete.  This approach minimises memory copies between the sublayers to only hardware link I/O (if required) and the segmentation and reassembly of data to or from ATM cells.

Since each cell.buffer may only have one reader and one writer, at most one SAR sender and receiver is allowed, though there is no restriction on the number of CPCS processes connected to SAR.  In fact, this is taken advantage of by placing CPCS stages on each cache processor, which places the transfer control stages in the middle of CPCS and above the AAL-SAP.

Figure 4.7: Process Diagram of AAL Implementation

### AAL Receiver

The SAR receiver requests a number of cells from the incoming cell buffer and identifies contiguous sections of ATM cells belonging to the same transmission.  Any non-user cells should be directed to a management layer, but in its absence, are currently discarded.

Once this has been done, a pointer to these cells is passed to the CPCS receiver so that reassembly into an SDU buffer can occur.  Since there may be several interleaved incoming transmissions, the CPCS is capable of reassembling into several SDU buffers, one for each incoming transmission.  Once the end of a CPCS-PDU is detected, the SDU buffer pointer is passed on to the AAL receiver.  The CRC check after reassembly may be omitted for video data if the performance penalty is prohibitive, though video recording remains largely unimplemented at the time of writing.

The CPCS-Receiver maintains a table of active streams, and requests a single SDU buffer in advance of a new stream arriving, such that the overhead of fetching a new SDU buffer for a new CPCS-PDU is avoided.  Any table entries that remain incomplete after a set interval of time are flushed from the table and their data discarded.  This prevents receipt of partial AAL5 packets from permanently consuming buffer resources.

Figure 4.8: Process diagram of AAL Receiver Implementation for ATM TRAM

Since the cost of inter-Transputer communication is relatively high, each processor involved in AAL is given its own protocol layers, rather than arbitrarily choosing one processor to deal with the higher layers.  If one processor were used for protocol processing, the recovered packets would then need transmitting to the target TRAM, which is expensive in link terms.  Duplicating the protocol layers across all subsidiary TRAMs has the disadvantage of increased code size on each.  However, it has the advantage that they all look and behave the same way, and further that all of the processors are used in the data processing, which produces a faster implementation.  Each subsidiary TRAM therefore acts independently of all others, and communicates with other Transputers only where a central repository of information need be consulted, for example, the file system data cache management structures.  The above design is summarised in figures 4.8 and 4.9.

For any incoming data, it cannot be predicted when data will arrive, nor how much there will be.  With any non-data packets, it is irrelevant which of the subsidiary TRAMs deals with the request; the CPCS.rec.primary (figure 4.8) may make an arbitrary decision.  However, when dealing with data blocks destined for the cache, it becomes important which TRAM is given the data.   In the case of the server's cache, a single cache block must be allocated to receive the data.  For example, consider a data packet that has already been received into an SDU buffer on a cache TRAM for which it was not intended.  Here, the packet needs either to be moved, or reallocated a cache slot on the TRAM concerned.  Consequently, the decision as to which TRAM should accept the data must be made before the hardware link transfer occurs.

The CPCS primary receiver delegates the job of link transfer to a link transfer control process, which itself delegates the responsibility of communicating over the hardware link to a link server, such that neither process blocks.  This arrangement is shown in figure 4.10.  For normal non-data traffic, the link control and server processes make an arbitrary decision on destination.  In this case, the transfer occurs into an SDU buffer on the chosen target TRAM and is subsequently processed by the relevant protocol stack, namely IP.  For record data traffic however, more specifically video data, the link server must be given a cache address into which it can make the transfer.  This address can only be given to it via the link transfer controller, which must obtain the addresses from the cache manager.

Figure 4.9: Process diagram of AAL Receiver Implementation for Cache TRAMs

### AAL Sender

In the sender case, the CPCS process attaches an aligned trailer, which includes a CRC, and submits the generated PDU to SAR.  The CPCS is therefore placed on the cache TRAMs, leaving only SAR-sender on the ATM TRAM together with a link read-transfer control process.  The transfer controller is attached to similar but independent link servers as the receiver's link write-transfer control process; this allows both unidirectional links to be engaged simultaneously.  A diagram of the read controller is omitted, since it is almost identical to that given in figure 4.10.  The sender's link transfer control process accepts the requests of multiple CPCS processes and those of the stream control processes.  Hence, the ATM TRAM is responsible only for the receipt of full CPCS PDUs, their segmentation into ATM cells and their subsequent transmission.  The cache TRAMs are in charge of the creation of the higher protocol layers from and above CPCS.

Figure 4.10: Hardware Link Transfer Engine of ATM TRAM AAL Implementation

A modification of the above scheme was considered where the cache TRAMs performed the segmentation into cells prior to transmission to the ATM TRAM.  In this scenario, the ATM TRAM would then merely manage the sequencing of transmissions and should be capable of sending onto the network at the rate at which the Transputer links could provide that data.  However, the effective data efficiency would then be 75% since both the cell headers and the cell data would be transmitted over the links.  Given that the achieved aggregate link-bandwidth was around 40Mbps, such a scheme would at best achieve little over 30Mbps.  In addition, transmission latency due to the segmentation time required before each link transfer would also be incurred.  It was felt at the design stage that the performance of the server would be better whilst performing segmentation on the ATM TRAM.

#### SAR Sender

The SAR sender process obtains ATM cell buffers from the cell.buffer process, and CPCS-PDUs from the CPCS, or more accurately, the AAL read controller process.  The PDUs are segmented into ATM cells and the appropriate headers generated for the cells.  The SAR should theoretically be able to deal with up to n simultaneous streams, where the streams the SAR has obtained are cell interleaved for transmission.  In this implementation, the value of n is effectively set to 1.  The SAR-sender delays forwarding of transmit cells to cell.IO where more data is expected, for example, where a next block of frame is to follow; this reduces process talk with the cell.buffer thus decreasing CPU scheduling overheads and increasing over all data throughput.

#### The Sender's Link Transfer Controller

Requests to transmit include a cache address list or an SDU buffer address, and the destination via a VPI/VCI pair.  The requests are placed onto a transmit queue inside the link transfer control process.  The data blocks are obtained from the cache TRAMs using the same mechanism as the receiver, but this time reading from instead of writing into the cache or buffer space; these blocks are forwarded to the SAR process for segmentation.  To implement the interleaved transmission mechanism required by SAR, the incoming requests are queued based on their time of submission; blocks are then sent from the first n frames on the queue.  This can be seen in figure 4.11 where the frames in transmission are those in the active queue. Transmission ordering is achieved by ensuring that all members of the active queue have unique VPI/VCI destination identifiers.

Figure 4.11: Process Diagram for Frame Transmission on ATM TRAM

The frame queue's block transmit requests feed into four link ‘block queues’, each corresponding to one of the cache TRAMs.  The frame transmit requests come directly from the streams manager process, which is discussed in section 4.9.  Each of the block queues additionally takes input from the cache TRAMs directly for the input of AAL5 packets generated by the IP processes.  The read control process can then take transmit jobs from the queues as the links become free from previous jobs.  Each link can have at most two jobs active at any instance (i.e. is double buffered) such that each link always has work to do as each block link transfer finishes.  As shown in figure 4.11, each hardware link has its own link server, multiplexor and demultiplexor process.  The link servers are essentially buffers to allow the main read controller to continue to allocate jobs across the four links without blocking on the multiplexor or demultiplexor.  In addition, the link server supplies the destination SDU buffer address to the demultiplexor before the remote cache TRAM transmits the block of data.  Thus, the data block can be received as soon as it appears on the hardware link without the additional temporal overhead of fetching the destination address.  A cache TRAM receiving a transfer job will simply transmit the indicated data block section onto the hardware link.  Once a block has been successfully received from a hardware link, the demultiplexor acknowledges an internal process of the read controller, which then causes several events:

- the block to be forwarded to the SAR-sender process;

- another transfer job to be allocated; and an acknowledgement forwarded to the frame queue such that the next block of that frame may be appended on the appropriate block queue.

The read controller, in addition to allocating transfer jobs to the four hardware links, is responsible for obtaining SDU buffers into which to receive the data.  In order to maximise efficiency, eight SDU buffers are fetched once the last buffer is exhausted; this minimises communication with the SDU buffer controller, reducing processor scheduling overheads and increasing process efficiency.  In addition, to maximise the efficiency of usage of SDU buffers, up to two blocks of each frame are placed into a single SDU buffer, which again decreases process talk with the SDU process.

#### Block and Cell Buffers

Since the sender may block when the cell buffers become full, the majority of memory is given to the receiver's cell buffers, since a lack of buffer space would result in discarded cell data.  However, enough transmit cell space must exist to allow the effective double buffering of cell transmission and cell preparation by SAR-sender.  This is also true of the SDU buffers where the receiver should always have a buffer available for reassembling a new block, but the transmitter must have enough buffers for the double buffered receive of data from all four cache TRAMs.

As the sender and receiver share the same SDU.Buffer process, it is important that the sender does not starve the receiver of reassembly buffers by filling them with blocks to transmit.  Care is taken however, to prevent the receiver from consuming all buffers and consequently deadlocking the transmitter.  This is achieved by limiting the number of buffers that are used for both transmission and receipt of data.  A design alternative was to use separate transmit and receive buffer spaces.  However, this approach was rejected because it makes less efficient usage of physical memory space, which is at a premium on a 512KByte TRAM.  The limiting of buffer allocation is achieved by allowing only transmit side processes to allocate the last n buffers, which are reserved.  Hence, the receiving processes cannot fill all system buffers with incoming data and cause deadlock in the system by preventing transmissions.

### Analysis of AAL5 Performance

Despite a great deal of effort to improve the performance of the AAL5 implementation, it remains a bottleneck in the server.  In particular, the need to segment and reassemble cells between the SDU buffers and cell buffers, and despite usage of the dedicated instruction MOVE2D, it is still a major consumer of CPU time.

Nonetheless, the maximum sustainable throughput currently lies at 28Mbps for outgoing stream data.  This is achievable with ideal conditions where transmitted frames are both large and each frame is block aligned such as with CBR data.  This is because the overheads per byte of transmitted data in setting up the data transfers are then minimised.  The typical maximum sustainable transfer rate, particularly with VBR data, is approximately 26Mbps.

Removing the segmentation/reassembly instruction allows the link utilisation to improve markedly to well over 40Mbps from a theoretical maximum of 50Mbps, part of which will be consumed in setting up the block data transfers.  Regrettably however, the CPU is not fast enough on segmentation to maintain this level of throughput.  Nevertheless, there is sufficient bandwidth available from the existing implementation to enable a small scale video file server, allowing for up to 18 of a desired 20 1.5Mbps streams, or more for lower bandwidth streams.

## A File Format for Continuous Media

A format for the real-time data stored on the Video File Server’s filesystem needed to be specified such that appropriate mechanisms could be built which manipulated them.  However, as outlined here, there were many ways to achieve the same end, each with its own advantages and disadvantages.

As the server is based around an ATM network and a file system with a zone size of 4 Kbytes, it is most sensible to enforce a maximum CPCS-PDU of that same size.  Thus, the unit of both transmission and storage is the PDU, greatly simplifying the management of the associated structures, not to mention having huge gains from not needing to generate the CPCS-PDU CRC's in real time on playback.

It is assumed for any data recorded that the same data and timings are reproduced exactly on playback.  It is therefore necessary for the server to generate an index containing timing information together with length and offset information into the recorded data.  Such information, however, cannot be embedded in the data PDUs because firstly the CRC's would be invalidated, and secondly, examining the meta-data in support of random access would require reading of the data blocks, which is very slow.

### Visible Versus Hidden Format

The following approaches were considered during the design phase of the file format:

| A. | Hidden Index: One possibility was to modify the existing file system such that the metadata structures used by the file server for stream activities were hidden from the user. While simplifying the appearance of files to the user however, such an approach precluded their implementation or manipulation by any system other than the file server. |
| --- | --- |
| B. | Index Attribute: A similar approach to A is to make the sequencing data a visible attribute of the video file as possible with NTFS [Cus94], in the same way that the filename and file length are attributes. However, this had the disadvantage of requiring major modifications to the existing filesystem structures. Furthermore, such files would be difficult to store on standard file systems, hence preventing general manipulation tools on some systems, i.e. those that cannot read or otherwise obtain the sequencing attribute. |
| C. | Appended Index: An existing solution by Henshaw[Hen95] places the sequencing information at the end of the main data file and stores a pointer to it in the file header. While this solution is practical, it suffers during recording because the header and index cannot be written to the file until the recording is stopped. |
| D. | Visible Index File: A better solution than C is to keep the header and index independent of the recorded data, giving each the same name stem, but the index a different extension. A convention was adopted that file header and sequencing data have the extension “.tim”. The file data itself is not constrained to have any particular extension so that families of differing media may coexist in the same directory, for example, aeroplane.vid, aeroplane.vid.tim, aeroplane.audio, and aeroplane.audio.tim. |

In taking the above approach D, the format is fully visible to the user, but has the big advantage of being portable across any file system and hence enables manipulation by any workstation with the appropriate tools.  This was essential here, where the first working prototype to be tested was a play only single stream server, which required the preparation of material for playback on a workstation.  Further, it is feasible that this format be used as a general ATM media format, not necessarily tied to the file server concerned within this thesis.

### Storage of Real-Time Data

With respect to the media data, each unit of transmission is stored encapsulated within a CPCS-PDU.  This is done such that playback of the file does not require the generation of a CPCS-PDU CRC and hence reduces the CPU load and the transmission latency as a consequence.  The MTU (Maximum Transfer Unit) for media transmission was chosen to be the size giving the largest throughput into the client workstation and being efficiently stored in a disk zone, in this case 4096 bytes.  As all AAL5 transmissions are formed from 48 byte cells, this effectively gives an MTU of 4080 bytes and a payload size of 4072 bytes.  Consequently, 16 bytes are unused in each 4096 byte zone giving a 0.4% space efficiency loss.

The following table outlines the options that were considered for the storage of media within the file-system data zones:

| P. | Zone Aligned Storage: As the file server's cache stores disk zones, each ‘start of frame’ must be zone aligned; the CPCS-PDU cannot lie over a zone boundary and one zone must belong to one frame. In this approach, two frames never share a single file system zone. |
| --- | --- |
| Q. | Shared Zone Storage: For a typical frame of compressed video, the size of the frame is almost never a multiple of the PDU payload, so the last PDU is mostly partial, i.e. doesn't have a 4072 byte payload. Consequently, an average of 50% of the final PDU of each frame is wasted in return for a transmission mechanism with very low latency. It is possible however, to reorganise the file structure so that the first PDU of a frame need not be of the maximum payload size. This would allow it to share a disk zone with the last PDU of the previous frame. In this way, the space efficiency is increased, but one more PDU per frame may be transmitted thus increasing the latency of transmission per frame. On the other hand, disk I/O requirements are potentially reduced, resulting in better cache performance because the file system must always transfer data in the unit of zones. |

The simpler approach P above was taken to trade off space efficiency for time efficiency in the first instance.  For the initial implementation, the simpler zone aligned approach allowed a working system in the shortest time.  Whatever choice made here, there are impacts upon the structure of the index of the recorded data, as discussed in the following section.

### Generation of Index

In storing the temporal information about data received, the two options for organising the index are either by PDU block or by frame.  During a recording session, it is difficult to arbitrarily identify frame boundaries without prior knowledge of the stream behaviour. For transmission, guarantees can be made about ordering of the data PDUs in either case, but there are other trade-offs, which are covered below.

#### Index by Block (Option R.)

This simplest approach assumes no association between data blocks.  The index is organised on block offset and stores in each entry the timestamp and total.PDU.length in bytes, as illustrated below. Each 32-bit field is little-Endian byte order such that the Transputer can access them as fast as possible.  In each 4K index block then, 4096  (2  4) = 512 entries per index block.  If a typical frame size of 5  4Kbyte blocks at 25 frames per second is assumed, 5  25 = 125 index entries per second is required.  Essentially, this is ¼ index block per second of data – one index per 4 seconds of data.

| 32-bits | 32-bits |
| --- | --- |
| Timestamp / s | Total PDU Length / Bytes |

If the more compressed shared-zone format of data is used, then an extra offset field is needed for each index entry.  The offset allows the frame start position within a zone to be located; this latter arrangement is illustrated below.  In each 4K then, there are 4096  (3  4) = 340 blocks per second.  Taking the same video example as before, the packed version is 33% worse off, but far larger savings are made within the main data file.

| 32-bits | 32-bits | 32-bits |
| --- | --- | --- |
| Timestamp / s | PDU Length / Bytes | Frame start offset/ Blocks |

Although this scheme appears to assume no relation between adjacent blocks, there is a need to calculate a relationship when streams are modified in either position or play direction; play must be restarted at the start of a frame and replay the blocks of a single frame in the correct order regardless of the direction of play.  Doing this on playback is not ideal because the system response time is critical, which is a serious disadvantage to this scheme.  Frame boundary calculation is discussed below.

#### Index by Frame (Option S.)

In observing the time distribution of blocks arriving it is possible to identify clusters of blocks which logically form single ‘frames’.  The exact characteristics of such distributions could be calculated either during the recording process, or as a post-process.  To be able to form frames during the recording would require some knowledge of the behaviour of the media, i.e. some parameterisation by the user.  This is difficult to do because different media streams have wildly different properties; it can be difficult to predict what these parameters are precisely enough.  Relying on the position of partial PDU's as a way to find frame boundaries does not work for streams where they do not occur, e.g. audio streams, or when the final frame PDU is perfectly aligned.  Nevertheless, the technique may be used as a hint coupled with timestamp based mechanisms.

A better approach is to record using the block index option R and reduce it to a frame index once recording has stopped.  Thus, a global view of the data distribution on the time line is achieved, which may be reduced by intelligently calculating the frame boundaries based on timestamp distributions and the position of partial PDU's in the data stream.  Using the block index during recording for later analysis, however, requires more memory than a pure frame index and the cache memory is consumed much faster.   Nonetheless, it is feasible that a recording block index be flushed onto the disk and retrieved later for the post analysis, in which case memory consumption during recording is not a problem.

The main advantage of organising the index block by frame is that on playback, no time need be spent working out where the start of a frame is and ensuring that all blocks of the frame are transmitted; the information is implicit in a frame index.

For each entry in the index block, the required fields are timestamp, total.length and block.offset.  If using the packed version of the data format, three words are still required, since block.offset becomes byte.offset from which the block offset can be inferred.  In each 4K index block then, 4096  (3  4) = 340 entries per index block.  Consequently, 340  25 = 13.6 seconds of video per index block are achieved.  This is a clear improvement over a block index, especially for files with large frames.  In the degenerate case where 1 frame = 1 block, frame indexing is 66% space efficient compared to non-packed data with block index, or as efficient in the packed case.

### Manipulation of Index in Memory

Because the stream manager process uses the same mechanisms to read the index blocks as it does the data blocks, the index is broken into a number of zone sized blocks.  Once these blocks are made memory resident by the cache manager, they need to be linked together such that the index blocks may be ‘walked’.  There are two main approaches to do this, as outlined in the next sections.

#### Index Block Chaining (Option Y.)

Two words of each index block are reserved as pointers to the previous and next blocks in sequence such that a doubly linked-list of index blocks is formed in memory.  The stream controller need only store the index block offset of the first memory-resident block and a pointer to that block such that a contiguous extent of the index is formed in memory.  Each index block therefore allows 511 blocks or 340 frame entries, and the above examples remain largely unaltered.

While the approach is easy to implement, for very large files the index tends to become relatively large as demonstrated above, although that problem is alleviated where some of the index may be paged out.  Unfortunately, sudden changes in stream properties may trigger a need for the entire index block, which will adversely affect system performance, i.e. in the case where the media file is being accessed at both of its temporal extremes.  In the ideal case, the index should be permanently resident until there are no more clients of the media file concerned.

A big advantage could be gained from having the stream server store the memory address of the last index block searched so that subsequent timestamp searches may occur from that index block.  However, there is no guarantee that the index block will be in the same place twice, i.e. where it was swapped out.  Such an approach therefore demands that the index be made fully resident and never paged out.  In some implementations, this may not be practical, forcing a fresh search of the index chain.

#### Index Page Table (Option Z.)

For media files with large indices, it is desirable that only those sections of the index that are being used are memory resident.  In this case, it is possible to indirect via a page table that has as each entry a pointer to a memory cache block, and the last timestamp of that index block.  This scheme allows searching of the complete index without needing to page parts of it in.  This approach is better than block chaining in the case where a stream's position is altered to a point where the required index block is not resident.  Here, the chaining method requires reading the entire index up to the point that is required.  In that case, the delay for a continuation of the stream will be less for the page table, but it is difficult to predict by how much in advance of construction.  The above hour clip example would suggest a possible delay of up to a half a second with a 2 Megabyte/s disk.  In any case, it is preferable to make resident the entire index, if possible, such that this problem is not an issue.  Indirection via a page table would also allow slightly faster search times on the index block for a timestamp because there is no chain to negotiate.  Although the same timestamp comparisons would be made, there is scope for a smarter search algorithm on the page table.

The last word of each page table is reserved as a pointer to another table.  If each page table entry takes 2 32-bit words, and each block index contains 512 block entries, 512  4092  (2  4) = 261632 block entries are achieved, which covers a 1 Gigabyte file.  Similarly, for a frame based index we achieve 340  4092 / (2  4) = 173740 frames, or 115 minutes of video at 25 frames per second.

### A Comparison of Indexing Methods

From the above discussion, it seems clear that the frame-based index is better on the grounds that it has no overhead in forming frames on playback.  The block to frame reduction penalty is paid only on recording, and the resultant index blocks are much smaller by a factor dependent on the size of frame.  As the framing process has to occur regardless of the method adopted, this cannot be used as an argument for elimination.  Thus, the server uses the frame-based index (option S) for playback files.

In respect of packing the media data, it made sense to include the option in the file index such that this alteration became a trivial step later.  In the first instance, the block byte offsets could be set to zero.  Hence, option Q was taken in principle, with initial tests using zone aligned frames.

Taking the example of a 1½ hour clip, the index required is 1.5  60  60  25  340 = 400 frame index blocks, a meagre 1.6 Megabytes.  Considering that the requirements are for far smaller clips, paging of the index block to disk is not necessary and the index block can be maintained in memory while a media file is busy.  In this case, the only argument between choosing index chaining or a page table is the time to access.  Here, the page table wins because there is no chain to follow prior to searching on the timestamps; the search may start at the point in the table that was last referenced.  In addition, the exact index block required can be located through the page table without needing to read in any of the index blocks.  However, these gains would only be apparent for clips of significant size.  Hence, option Z was taken for the server file format.

For the current requirements, the advantages of a page table will not be particularly significant, but it does allow for expansion later.  At the very least, it would not be worse than block chaining and previous experience would suggest planning for the future.

### ATM Media Header Content

Enough information must be placed into the header to allow:

| i) | Identification of the file type. A signature is placed into the header such that an ‘alien’ file cannot be mistaken for a media file. The text string “ATMMedia” followed by the arbitrary hexadecimal number AE9422BF shall be used. |
| --- | --- |
| ii) | Version number of file format (1) and index type identification (block. vs. frame) |
| iii) | Information about the data blocks. Storage of the total number of blocks, total number of frames, and the total duration of the media. |
| iv) | Information on the data format. It is a requirement that the compression format used on the data be independent of the ATMMedia format. Therefore, an arbitrary piece of information is placed into the header that is meaningful only to the client application. For example, a JPEG format identifier and the JPEG compression parameters. |

The above is summarised in figure 4.12.

A total of 4KBytes is reserved for use by the header.  This is followed immediately by the index page table(s) and index blocks, respectively.  The number of index page table blocks present can be deduced from the Block.Size and Total.Frames or Total.Blocks.

In previous calculations it was assumed that one video file would occupy one filesystem f-node.  After the design of the file format was completed, it transpired that each ATMM file would consume two f-nodes, so a recalculation of required f-nodes in the filesystem was performed to maintain the maximum required file capacity.

Figure 4.12: Format of the file header.

### A File Format for Continuous Media – Summary

To allow maximum versatility and future expansion, the Continuous Media File Format uses a frame based index with page tables.  The real-time recording of data should use a block based index which is reduced to a frame index as a post-process, though no stream recording has, at the time of writing, been implemented in the server.  The header and index use the “.tim” convention with the main data taking the same name stem.  The main data may occur in either compressed or non-compressed versions with the frame index allowing either version via using the unit of Bytes for its data offset field.  The server can, in theory, record in either compressed or non-compressed formats where two consecutive PDU transmissions are no larger than a cache block.  Conversion between these two formats should be possible with an appropriate tool.

For the purposes of future expansion, the server file format was modified such that the 5 least-significant bits of the frame transmission timestamp were reserved.  The purpose of the 5-bit field was as an inter-frame priority for use with, for example, MPEG clips.  A mechanism to clip to a 0-priority frame after a user-reposition, or during fast playback, has not been attempted with the prototype server.

## Management of the Data Cache

The cache is divided into blocks of a fixed size, equal to the file system block size.  The cache addresses are arranged such that the two least significant bits of a 16-bit wide address are the cache offset number (0 to 3).  The remaining 14 bits form the block row index, logically shifted left by two.  Thus, the block ordering is from cache 0 through cache 3 at row address 0, then again at row address 1, and so on.  This addressing arrangement is illustrated by figure 4.13.

Figure 4.13: Diagram of 16-bit Cache Address

As the cache blocks are distributed across the four TRAMs, it is not possible to add information to each of them to enable their management; such a structure must be centralised for reasons of speed of access.  Additionally, to do so would break the contiguity of blocks within the cache TRAM; that would restrict implementation possibilities with respect to the maximum size of data transfers with SCSI and ATM devices.  Thus, the management structures are a separate entity.

The primary index of the management structure is a quantized cache block address, in this case, the row address.  This minimises the total size of the management structure and improves search times when looking for file blocks; each row of the cache is given only one management structure element.  The size of the management structure for a 90Mbyte cache is approximately 112Kbytes, which would be 450Kbytes were row addressing not used.  However, this reduction gives a cache size gain of 1.3Mbytes since all four TRAMs must use identical cache memory sizes when striping data in transfers.  Thus, the primary index of the management structure is the 14 most significant bits of a cache address.  The implications of this arrangement are that the file system must then retrieve a minimum of four blocks into the cache.  In fact, this is not a problem, since such a large transfer size (16Kbytes minimum) is required to maintain good file-system performance.  The management structure is shown in figure 4.14; the contents of each element are shown in figure 4.15.

Each file can be uniquely identified by a file handle, which can be used as a key when looking for blocks belonging to a particular file in the cache.  The structure elements allow the blocks of a single file to be chained together in sequence.  Similarly, the unallocated cache rows are chained into a single free-list.  The files active in the cache are stored in a most recently used order, such that eviction of files from cache space can be done on a least-recently-used basis.  The ‘down’ pointer of the management element is used only by the first block of each file, and is used to chain together the first block of all files.

Figure 4.14: Diagram of the Cache Block Data Management Structure

Figure 4.15: Cache Block Management Structure Element Contents

## Communications – TCP/IP and RPC

Once the file-system, network interface, and cache management were all in place, Andrew Smith’s UDP/IP implementation, from another project run in the laboratory [LIP95], was modified and used to give UDP and ICMP (including ‘ping’) support to the server.  With this complete, an RPC [RPC88] implementation was built on top of Smith’s UDP engine, and on that the Network File System (NFS) protocol [NFS89, NFS93].  The implementations of RPC (XDR [XDR87]) and NFS were based on specifications available on the Internet.  The completion of these steps enabled the connection of the prototype server with other network end-systems.  However, NFS support only provided file meta-data exchanges, and not file-data exchanges, which required further issues to be addressed.  These issues are covered in the next section.

## NFS I/O

This section outlines the possible approaches that could have been taken in transferring user data from the inbound network packets onto the disks, and from the disks into outbound packets.  Based upon the arguments presented in the main text, it concludes with the most appropriate approach to the given problem.

### NFS I/O Implementation Issues

The file system transfers data via the four Transputer hardware links to or from the connected cache TRAMs; these data are taken directly from or directly to the disk with no caching structures in between.  For each zone of the file system, there is a requirement of one address corresponding to a block on one of the cache TRAMs, where the block is the same size as the disk zone.

When the file server receives an NFS WRITE call, the SDU buffer containing the CPCS-PDU is arbitrarily transferred to one of the cache TRAMs for IP processing.  It is at this point that the NFS call is identified and herein lies the problem: by the time the WRITE call is recognised, the data to be written is already on one of the cache TRAMs, and furthermore, the data to write is not in cache-block space.  A mechanism needed to be determined by which the IP data could be manipulated into an appropriate form for writing onto the disk.

For an NFS READ call, the relevant disk blocks can be fetched into cache blocks, but these are distributed across several TRAMs, and it is possible that none of those TRAMs will correspond to that of the responding IP TRAM.  The question that needed to be answered was ‘how can data be gathered from one or more Transputer address spaces into a single outgoing IP packet?’

As can be seen, the READ and WRITE cases are subtly different from one another, and are thus dealt with separately.

### The NFS READ Call

On the occurrence of a READ call, the file blocks required may or may not be resident in the cache.  In the latter case, the cache manager will fetch the required blocks before identifying them to the IP process dealing with the READ.  It is assumed that the READ will always go via the cache, rather than reading data directly from the file system and thus defeating the purpose of the main cache.  In generating the response to the call, there are the following possible approaches:

| A. | The required sections of the cache blocks are requested from the appropriate cache TRAMs and transmitted to the IP process for assembly into a single outgoing packet. Therefore, the cost of transmitting data between the cache TRAMs will be incurred, which will occur ¾ of the time assuming a uniform distribution of data across the TRAMs. Of these transfers, one to three TRAM hops to the destination will be needed, the worst case with the IP responder at the end of the cache chain. It was anticipated that such an approach would be prohibitively slow and inappropriate to a real-time environment. |
| --- | --- |
| B. | Each segment of data to transmit is identified by the IP process, and a command transmitted for each segment to generate an IP packet for each part, i.e. IP fragmentation is used. This has the advantage of avoiding inter-cache data transfers and will be faster than case A. Furthermore, as IP fragments have a sequence number, the order of transmission becomes irrelevant and the time of response therefore potentially quicker. The disadvantage of this scheme, however, is that a memory copy from the cache block into the SDU buffer that the returning IP packet is prepared in is still required. A further development of the scheme is thus to lock down the cache block until transmission is completed, and therefore to transmit the data in-situ and avoid the memory copy. This latter approach would complicate the CPCS-PDU generation as the CRC would need to be taken across two, not one data segment, and would require modification of the process handling I/O to the ATM TRAM for the same reason. |
| C. | Similar to option B, but partial CPCS-PDUs are transmitted to the ATM TRAM rather than IP fragments – the streaming mode AAL service. As the CRC of each fragment is dependent upon all previous segments, a sequential processing of the data segments to transmit is enforced. Consequently, despite its lighter weight as compared to B, it may be slower if parallel processing cannot be taken to advantage; ideally, a way to calculate a single block CRC from segments of the block concerned should be done in parallel, and the part CRCs combined. A further limiting factor is the need to ensure sequential transmission of the parts as ordering now becomes important; there are no implicit sequence numbers. The inherent sequential ordering imposed by the CRC calculation goes some way to solving this, but it is possible that the ordering could be broken where some of the ATM TRAM hardware links were busy with non-related transfers. However, the default IP MTU for use over ATM is 9188 octets (8 Kbytes data payload). If it is assumed therefore that a single IP packet is at most 8K, then this can be spread over at most three cache blocks/TRAMs and the problem is finite; the order of arrival should be as intended without any ordering function for even very high loading conditions. The current implementation of the SDU buffer transmitter maintains four independent queues of requests and it services them as the links become free. Any raw AAL5, i.e. video data transmissions, will take precedence over the IP transmissions. Thus, where one to three of the hardware links are saturated with AAL5 data, the remaining ‘free’ links will allow the SDU based transmissions to get ahead on the respective TRAM. For this reason, ordering of SDU based transmissions cannot be guaranteed without modification of the transmission mechanisms. For example, one modification would be to use a single SDU queue and service it sequentially. Although this would solve the problem, the parallelism of the transmitter would be impeded and resultant throughput likely diminished. |

### The NFS WRITE Call

There are two approaches to copying the IP data payload onto the disk.  In the following, it is assumed that NFS writes may be of arbitrary byte size and write offset:

| A. | The file system I/O mechanism could be modified such that a section of data within an SDU buffer can be transferred directly into the SCSI TRAM. As the file system itself is block based, a READ must first be executed so that the data can be transferred in at the correct point of the block, and then written to the disk. Such a read is not required where the write is both block-sized and block aligned. Consequently, each write operation always bypasses the main cache and is a read/modify/write where the data is not block-sized and aligned. In addition, as the data to write is not distributed across the cache TRAMs, there is a lower maximum throughput in writing the data to the disks; the file system is capable of engaging all four TRAMs where the data to transfer is striped across them. |
| --- | --- |
| B. | To use the current file system I/O mechanisms, the IP data payload must be transferred inter-TRAM such that the data to write then exists at the correct point within the appropriate cache block. It is clear then, that a penalty similar to READ case A is incurred. However, by using cache blocks it is possible that the block of the file concerned is already in the cache, and there is no need to READ in the full block from the disk (read/modify/write). Secondly, by using the cache it is possible to defer the time of a write and hence increase throughput by reducing the disk load. The initial disk READ is obviated where the payload occupies the entire (non-resident) cache block. A further advantage is that the I/O mechanism of the file system would not require any modification to make this approach work. |

### NFS I/O Implementation

The efficiency of an NFS read was not a critical component of the server.  Further, considering the either non-existent or unpredictable nature of some IP fragmentation implementations, the case C READ with CPCS-PDU fragmentation was seen to be the most appropriate solution.  On the assumption that most random file accesses are for small amounts of data, then data fragmentation should not occur frequently, the worst case being two fragments where the requested data straddled a zone boundary.  In practice, NFS read requests are always block aligned, and hence this latter situation does not in fact occur; the server can nonetheless cope with it.  With respect to enforcing the ordering of transmission, a split transfer (that is, across two or more TRAMs) is marked as such, and the TRAM that will transmit the next segment of data is declared to the ATM TRAM.  In addition, to avoid confusing two or more such split IP transfers from a single IP TRAM, the partial block CRC is used as a transfer identifier.  The CRC is used to both identify and validate a transfer stream.  Namely, where a mismatch occurs between the last-CRC given to the ATM TRAM and the last-CRC offered by the next cache TRAM to transmit, the transmission is aborted.

In the WRITE case, there is a potentially significant gain to be had in operating through the cache.  However, on the assumption that NFS writes are a rare occurrence, such a gain would be largely insignificant.  In any case, with the file server optimised for reading, the time taken for the write to occur is less important and the simpler solution to write-through the cache is the better one.  Given also that actual NFS implementations transfer in block sized units, the performance to the server’s writes should at least be reasonable.  Thus, option A was selected for server NFS writes.

To improve the NFS throughput performance of the prototype, a work avoidance technique was applied as suggested by Juszczak [JUS89].  For example, if a client requests a block of data from a heavily loaded fileserver, the client may retry the operation because of the high server latency, perhaps several times.  The server therefore has the original request and one or more duplicates within its processing queues.  Thus, the server in fact replies more than once to the client for a single logical operation, which further slows the server and in turn causes more requests to be repeated.  The technique suggested by Juszczak produces an improvement in server bandwidth by recognising and eliminating duplicate client requests that have already been processed, and hence allows the server to allocate resources elsewhere.

## Analysis of IP and NFS Implementation

Initial testing was based upon trying simple file operations to exercise individual NFS and file system primitives, which enabled the server to become operational.  To more thoroughly test the correctness and efficiency of the server’s NFS implementation, the NFS benchmark [SCW88] was later employed with great success.  This allowed the validation of the file system under heavy loading conditions, with the benchmark checking that its operations were correctly carried out.  In particular, no abortion of split IP (i.e. fragmented CPCS-PDU) transfers occurred at any time.

In table 4.1, the machine ‘cobra’ is a Hewlett Packard Apollo 725/50 running HP-UX version 9.  The machine ‘python’ is an HP Series 700, also running HP-UX 9.  Both of these machines have 64Mbytes of RAM.  Thus, these test machines are high power workstations and consequently give excellent NFS performance.  The machine named ‘sequoia’ is a Sun IPX SPARC station with 16Mbytes RAM, running Solaris 2.5.1.  This latter machine differs in its NFS implementation in that it uses an 8Kbyte block size for data transfers.  This has a clear effect on the prototype server’s NFS performance.  The control data is taken from the original 1988 NFSstone documentation and is simply given for reference.

The two prototype servers, vfs-atm and cimis-vfs, were configured in these tests to use the same hardware and cache size.  However, cimis-vfs uses a single 7200rpm disk compared to the development prototype’s two 2100rpm disks.  The benchmark results are therefore a clear indicator that a faster disk will significantly improve server performance.

Table 4.1: NFS Benchmark Results for 45522 NFSstones

| Hosts |  | Benchmark |  |
| --- | --- | --- | --- |
| From | To | Time / s | NFSstones / s |
| Sun3/60 v3.4 | control server | - | 85 |
| cobra | vfs-atm | 362 | 126 |
| cobra | cimis-vfs | 313 | 145 |
| python | vfs-atm | 351 | 130 |
| python | cimis-vfs | 309 | 148 |
| sequoia | vfs-atm | 205 | 223 |
| sequoia | cimis-vfs | 181 | 252 |
| cobra (Ethernet) | python | 168 | 271 |
| cobra (ATM) | python | 182 | 250 |

The prototype server is at a disadvantage as compared to the workstations, since each network packet must be transferred internally across at least one Transputer link.  This adds in extra receive and transmit latency, which the workstations do not incur.  In addition, the Transputers are responsible for CRC generation in the AAL5 packets, both for incoming and outgoing packets.  This is a feature performed in the workstation’s much faster ATM hardware.  The processors on the workstations are also a good deal faster than the Transputers of the prototype server, on the HP’s in particular.  Not surprisingly then, the workstations give better performance figures.  The 8Kbyte block-size of ‘sequoia’ effectively reduces the effects of receive/transmit latency, since half the number of I/O exchanges occur compared to the HP-UX boxes.  In particular, the server’s disks can achieve better write performance with larger block size. Consequently, the more modest Solaris workstation produces better benchmark results.

Notice that the figures for the Ethernet and ATM benchmarks between the HP-UX workstations are not significantly different.  This would suggest that the benchmark is not network-bandwidth limited.  Furthermore, the lower ATM performance between these machines would suggest an immature ATM driver.  Regardless, the implementation of the prototype is shown as correct, and its performance is very reasonable.

### Behaviour of the ATM hardware in the File-Server

An analysis of the ATM and AAL implementations could only usefully be performed with a server under heavy loading conditions.  Thus, the NFS benchmarks also served as a good environment in which to examine the behaviour of the cells output by the ATM hardware.  This analysis revealed that the ATM adapter was occasionally outputting two back-to-back cells, which is caused by a relative cell mode transmission being made long after half the hardware clock period since the last transmitted cell.  This discovery further indicated that the ATM hardware could stall if the first cell to transmit, which underlyingly uses an absolute mode transmit timestamp, is delayed.  Such delays can occur because of heavy CPU loading; the hardware will not transmit the cell for up to a further half clock period.  Hence, the implementation of immediate-mode cell-transmission in the ATM firmware should improve server performance for both NFS and streams clients.

## Implementing Stream optimised Caching

The diagram in figure 4.16 illustrates the process diagram for what is essentially the heart of the prototype server.  The processes again use the Occam ‘circle’ notation, but in 4.16, some circles have attached rectangles; these represent data structures, which are referred to in the text below.  Where a single structure occurs across more than one process, the processes involved share the structures, and do not have separate copies.

The use of a synchronisation mechanism occurs only once in figure 4.16, between the cache manager and the transmitter.  The sharing of data among processes is carefully managed such that expensive synchronisation mechanisms are not needed – it was the concern of the author that heavy use of mutual exclusion, for example, would quickly make the host Transputer CPU bound.

The following sections concentrate on the different areas of figure 4.16, which together make up the cache manager and streams control suite.  The operations of the cache and stream managers are closely linked, and are therefore discussed together.  Similarly, because of the close relationship between the NFS daemon and the cache manager, a discussion of how these components fit together has been deliberately deferred to the next subsection.

Figure 4.16: Cache and Streams Management Process Diagram

The edges of the diagram in figure 4.16 show the connections to the ATM and SCSI TRAMs.  In addition, the IP process is shown as one process, though its responsibility is split over the four TRAMs in the backbone of the cache.  For the prototype, the cache manager TRAM does not receive incoming IP traffic directly; the CPU is therefore dedicated to maintaining good real-time response during the operations of client streams.  Hence, the ukcStreams requests are forwarded to the manager TRAM via an IPC mechanism linking the four TRAMs.  NFS data requests are handled without forwarding the entire RPC packet to the manager TRAM, as explained in the following subsection.

### NFS and the Cache Manager

Each of the other cache TRAMs has an IP process which deals with traffic from the network.  In general, the IP processes deal with NFS requests by talking directly with the file system on the SCSI TRAM.  For data reads however, the cache needs to be used.  In this case, the NFS daemon requests that the cache manager load data into the cache and return the address, or addresses, of that data.  Of course, the data may already be present in the cache, in which case no data need be transferred from the filesystem TRAM.

The IPC mechanism connecting the processors on the cache backbone is used to make the data request to the cache manager.  In figure 4.16, the NFS proxy receives this request and forwards it to the cache manager.  Similarly, the list of addresses given by the cache manager is returned back through the IPC system to the calling NFS daemon.  Once the addresses of the data are returned, the return RPC packet can be constructed using the case C READ in section 4.8.2.

Since the cache is allocated in 16Kbyte rows, data read from the file system is limited to multiples of the row size.  In fact, this is a standard performance technique in file systems; the prototype uses a 32Kbyte basic fetch size.  In addition, the cache manager will ensure that the next 16Kbytes of data after that requested by the caller is held in the cache, since normal file data are typically sequentially accessed.

To further improve NFS performance, ‘cookies’ are used to reduce the time spent searching file chains.  Each file in the cache has its last accessed cache-row offset recorded.  Thus, for each access to a file, the cookie is used as the start point of the file chain search.  It is therefore important to maintain the file ownership of all lines in the cache management structures (figure 4.15), since the cookie might otherwise cause a search on the wrong file chain, or the free-list.  Since the majority of file access is sequential, the search of the file chain is either not needed (same cache row), or is of length one.

Cache space is divided between NFS files and stream files by counting the number of stream users of each file in the file header chains; any files with zero counters are normal data files.  NFS is allowed to occupy at most n Mbytes of space if the server has stream clients; in the prototype’s case, 4Mbytes is the maximum space given to normal files if the server has stream clients.

The inclusion of a stream counter within the head of each file in the file chain requires that streamed files never be flushed from the cache, since that would lose the client number state.  However, streams typically always have data in the cache, so the constraint is not a limiting one.  During group reassessment, the state counters make it easy for the manager to track resource allocation to normal and streamed data files.

The normal data files each have an expiry timer when streams are active.  If no file activity to the normal file is made within a set timeout period, then the file concerned is flushed from the cache, hence making way for stream data.  The prototype uses a timeout of five seconds.

If the cache becomes full, the cache will flush NFS data from the cache on a least recently used basis until enough space is available.  A full cache will also trigger a reassessment of stream groups, which will also cause space to be freed in the cache.

### The Cache and Stream Managers

The stream manager is responsible for maintaining the structures set out in Chapter 3, viz. figure 3.6.  Namely, it keeps track of which files have stream clients, and organises the streams into groups. However, the stream manager does not make any data transfer requests to the file system directly; any transfer requests are made to the cache manager.

The details of how the streams are formed into groups was given back in Chapter 3, and is not repeated here.  The prototype implementation is specifically a pure streams algorithm, and does not make use of the residual space left after allocating the cache to the smallest set of intervals.  The discard of material from the end of each stream group is passive, that is, is not removed from the cache until the streams are reassessed.  Reassessment occurs either when a client stream issues a reposition, a new stream arrives or another leaves, or the cache runs out of space.

The streams manager is driven by the incoming ukcStreams protocol requests, which are forwarded from the IP processes on the other three TRAMs on the cache backbone.  The stream manager is therefore the ‘ukcStreams daemon’, and is responsible for replying to the protocol requests.  Hence, the manager TRAM always responds directly onto the network to ukcStreams requests, and does not reply to the forwarding IP process.

The IPC mechanisms are used by the stream manager when setting up a connection with a client.  The file that a client specifies has its metadata checksum checked against the main data file.  Since the first file block is located in the first cache TRAM, an IPC request is made to it.  Once the checksum reply is made, and the checksum is found correct, the connection procedure is completed.

### Organisation of the Streams

For each CONNECT primitive received, the stream manager assigns a stream handler process to the maintenance of the client stream.  Each handler process maintains the logical clock of its stream, and is responsible for the transmission requests of the frames of that stream.  Thus, each handler has access to the meta data index of its charge.  The index is fetched by the manager as part of the client set-up call.

Once the manager has assigned a handler to the operation of a client stream, it will alter or interrogate the stream as appropriate, according to the ukcStreams primitives that arrive from the client.  The handler processes communicate with the manager process only through the ‘Alert’ queue, since to do otherwise would cause deadlock.

The alert message is either a prefetch request, or a cancel notification.  Each stream makes a prefetch request when the previous prefetch’s data has been half consumed.  The prototype tries to fetch a second of data ahead, and therefore makes requests each half second, even if the playback rate is altered.  If the rate of play is altered, this affects the amount of data fetched from the disk.  The prototype imposes a minimum transfer size of 32Kbytes and a maximum of 256Kbytes to keep performance high, while preventing one stream from consuming all of the disk bandwidth.  If the alert queue has a previous request from the same stream, the previous request is overwritten, since the server is presumably too busy and has not yet serviced the last prefetch request.

The cancel notifications are caused by timeouts in the handlers.  If there is no client activity for a paused stream for a specified period, then the handler tells the manager to free up the stream.  The timeout period is specified as part of the CONNECT call.

The stream manager services the prefetch requests in First Come First Served (FCFS) order.  At start-up therefore, all active streams will access the disk; the logical allocations of the stream manager made during assessment will slowly be turned into physical allocations.  Once stable state is reached, only the leading streams of each group will access the disk, with the prefetch requests of the others being absorbed by the cache.

The stream handler processes request frame transmissions to the cache transmitter process.  The transmitter, on receiving a request, searches the cache for the blocks requested.  If the data are found, then the request is forwarded to the ATM TRAM for transmission to the client.  As the cache manager can be adding or destroying file chains in parallel with the searches of the transmitter, mutual exclusion (a ‘mutex’) is needed to protect the transmitter from having the cache structures pulled from under it.

Similarly to the use of cookies with normal data files, the CPU time spent searching for file blocks to transmit is minimised by recording the cache location last accessed for each stream.  The search for a file block continues from the point indicated by the cookie, and search complexity is generally reduced to one frame.

### Flow Control and Deadlock Prevention

Should the ATM TRAM become congested, and the transmit queues back up enough to fill, the ATM adapter will switch off the flow of transmission requests.  The variable ‘transmit.on’ shared between the stream handlers and the transmitter, is checked by the handlers before any attempt to transmit is made.  This approach prevents the handlers from blocking on a request to the transmitter, because the transmitter is itself blocked on a request to the ATM TRAM.  The flow control is enabled once the ATM adapter is able to reduce its outgoing frame transmit queue to 25%.  The ATM TRAM will throw away requests that it cannot cope with, though this results in the transmitter process making requests that are promptly discarded.  The flow control mechanism therefore switches off the requests at their source and saves link bandwidth and CPU time for both the manager and ATM TRAMs.

### ukcStreams Protocol – Extensions to Palantir

During the development of the server, two other projects in the group made an attempt to play back synchronised audio and video.  The results were not perfect, with lip synchronisation in particular being poor.  Thus, the protocol designed for the prototype server included an ‘ASSOCIATE’ primitive, which slaves the clock of the client stream to that of another stream.  Since the server is responsible for the transmission of data from two or more streams from a single logical clock, the synchronisation between the two should be far better.  In addition, the slaving of the clock can be offset by a positive or negative amount, such that delays in processing different streams at the client can be accounted for.  Furthermore, the size of the offset can be altered ‘on the fly’.  At this time, the ability to associate streams has only been demonstrated between multiple video streams; it has yet to be used for the synchronisation of audio and video with lip-sync.

## Summary

In this chapter, the components of the video server were presented together with an explanation of the alternative approaches considered and the reasons why particular implementation paths were taken.  On analysis, each of the system components was shown to satisfy the initial estimate of required performance, and was therefore sufficient to enable the successful construction of a networked video file server.

A number of notable performance related lessons were learned whilst constructing the server.  These optimisations were necessary in order to achieve the required server performance.  However, a discussion of these techniques is deferred until the closing Chapter 7.

An extent is a contiguous arbitrarily sized fragment of a file.

See section 4.2.9 for a discussion of Volume Sets.

Although the time of one revolution of the disk is the same for any track on the disk, the outer tracks have more sectors than the inner tracks.  Hence, the quantity of data transferred per revolution is higher on the outer tracks; the data transfer rate is therefore greater.

Field Programmable Gate Array

Interrupt Service Routine

The first implementation had a maximum sustainable transmission rate of 17½Mbps

The length in bytes and the byte-offset of the PDU from the start of the file are

stored in the .tim index.

The author became aware of a technique to recombine partial CRCs calculated in parallel at the end of the SERVICE project.  However, no references to this work could be found.

IPC – Inter Processor Communication